Notable Computer Networks

John Quarterman

jsq@ut-sally.ARPA jsq@ut-sally.CSNET

jsq@ut-sally.UUCP ihnp4!ut-sally!jsq

84/07/06

Department of Computer Sciences

The University of Texas at Austin

Austin, Texas, 78712 USA

This is a list of  notable  computer  networks  in  the

United  States.  Its purpose is to acquaint people with what

networks are out there and to give some  idea  of  how  they

differ:  it is not intended to be exhaustive and it is not a

manual, just an indication.  Where sources are quoted verba-

tim they are credited.  Most such quotes came from a discus-

sion in HUMAN-NETS (as the ARPANET mailing  list  is  known;

fa.human-nets  to  USENET).   Most of the articles have been

shortened but are otherwise unchanged.

Where an access method from the DARPA Internet,  CSNET,

or  the UUCP mail network is known, it is listed.  There are

numerous networks outside of the USA (such as Canada's DATA-

PAC),  but  they are not listed here due to lack of informa-

tion or space.

The standard reference for the theory of computer  net-

works is still Computer Networks by Andrew S. Tanenbaum.

_________________________

Posted  to  net.sources   on   USENET,   available   as

networks.doc   or  networks.ms  from  ut-sally.ARPA  by

anonymous ftp (login anonymous, password guest).   It's

basically a handout for new users that some people have

thought might be of interest elsewhere.

July 6, 1984

- 2 -

network           type    ser-    hosts   speed/   address

vices           relia-   (user@host.domain)

bility

_______________________________________________________________________________

DARPA Internet   ?i T-d   lfm-o   800+     6/9     u@h.ARPA

ARPANET         RI T-d   lfm-o    180     6/9     u@h.ARPA

MILNET          MI T-d   lfm-o    188     6/9     u@h.ARPA

XEROX internet   Ci ?-2   lfm-o   ?000     6/9     u.registry@XEROX.ARPA

PDNs

ADP Autonet     P- X--   lfm--     ?      4/9     ?

CompuServe      P- X-d   lfm--     ?      6/9     ?

GTE Telenet     P- Xpd   lfm--     ?      6/9     ?

Tymnet          P- X--   lfm--     ?      6/9     ?

Uninet          P- X--   lfm--     ?      6/9     ?

CSNET            Ri Cp1   --m--    100     4/9     u%h.csnet@csnet-relay.ARPA

u%h.csnet@rand-relay.ARPA

CSNET/TELENET                                     u@h.ARPA

MAILNET          R- Cp2   --m--    15      ?/?     u%h.mailnet@mit-multics.ARPA

BITNET           ?- V-d   ?fm??    50      ?/?     u%h.BITNET@Berkeley.ARPA

DEC              C- D-4   lfm-o   2000     ?/?     u@h.DEC

u%h.DEC@decwrl.ARPA

UUCP

transport       A- Up-   -fm-o   2000?    3/4     not applicable

mail            A- Up4   --m--   1900     3/3     u@h.UUCP

ha!...!hz!h!u

USENET news      A? *p4   ---n-   1000     4/7     not applicable

Types:

administration

C: Commercial, i.e., internal to a private company

P: Public Data Network, i.e., a common carrier

R: Research (e.g. Defense Advanced  Research  Projects

Agency or National Science Foundation or University)

M: Military (e.g. Defense Data Network)

A: Anarchic (mixed commercial  and  research  with  no

central control)

internet

i: an internet (interconnection of  multiple  physical

networks using the same transport protocols)

I: part of the DARPA Internet  (the  oldest  internet,

except maybe XEROX's)

transport layer

T: TCP/IP

C: CSNET: TCP/IP (ARPANET);  TCP/IP on X.25 (TELENET);

MMDF (Phonenet)

X: X.25

V: VNET

July 6, 1984

- 3 -

D: DECNET

U: UUCP

speed

p: most links are ordinary speed phone lines

scope

d: domestic USA only (or mostly)

#: a digit indicating the number of continents reached

Services:

l: remote login; f: file transfer; m: mail;  n:  news;

o: other

Hosts:

The number of hosts given is as close an  approximation

as  could  be  found (84.03.28), but all these networks

are growing.  Certain networks, in particular UUCP  and

USENET, prefer the term site to the term host.

Speed/reliability:

The measures given are on a scale of 1-10 (low to high)

and  are  purely  subjective,  though  most people with

experience with the networks  involved  would  probably

agree with the relative rankings.

Address:

The addresses given are templates for ones  that  could

be used to reach a user on the indicated network from a

host connected to both the DARPA Internet (and  thereby

to  CSNET) and the UUCP mail network.  Gateways between

networks in certain domains are a touchy political sub-

ject which is not addressed for certain cases.

_#D_#o_#m_#a_#i_#n_#s

In an  address  like  jsq@ut-sally.ARPA,  ARPA  is  the

domain  name;  the name of the network _#u_#t-_#s_#a_#l_#l_#y is a part of

is ARPANET, not ARPA.

Domains  are  administrative:  they  do  not  refer  to

specific physical networks.  Domains are registered with the

Internet Network Information Center.  The only current offi-

cial  domain name is ARPA, which applies to all the networks

in the DARPA Internet, including ARPANET,  MILNET,  and  all

the  ethernets,  token rings, and other local networks using

TCP/IP and connected to either of the above networks through

gateways.  The DDN domain will appear in the next year or so

and will apply to MILNET and local networks connected to it.

The  UUCP domain, while not official, is widely used, as are

CSNET, MAILNET, BITNET, and DEC.

The domain scheme is not  fully  implemented  yet,  and

only  really  applies to mail at the moment where it applies

July 6, 1984

- 4 -

at all.  Details can be found in several ARPANET RFCs.

RFC897   Feb 84   Postel        Domain Name System

Implementation Schedule

RFC883   Nov 83   Mockapetris   Domain Names -

Implementation Specification

RFC882   Nov 83   Mockapetris   Domain Names -

Concepts and Facilities

RFC881   Nov 83   Postel        The Domain Names

Plan and Schedule

RFC822   Aug 82   Crocker       Standard for the Format of

ARPA Internet Text Messages

See below under DARPA Internet for how to get these RFCs.

_#D_#A_#R_#P_#A _#I_#n_#t_#e_#r_#n_#e_#t

The offspring of the old ARPANET,  all  running  TCP/IP

and  related protocols, connected through gateways, and with

a common name space.  There is a Network Information  Center

at  SRI  (host  _#s_#r_#i-_#n_#i_#c)  which currently keeps track of all

hosts, but plans are being made to divide the Internet  into

domains, each with its own name servers.

While the two main constituents are ARPANET and  MILNET

(each with about 180 hosts), there are numerous local ether-

nets, token rings, etc. at  universities  and  private  com-

panies  which  are  part of the Internet.  Frequently such a

local network will not have all its host names  listed  with

NIC,  preferring  instead  to  maintain  a mail gateway that

allows one known host to forward mail to others on the local

net.   Outsiders  must  somehow  just  know  the appropriate

internal address until domain name servers  are  functional.

This  makes the real size of the Internet hard to judge, but

there are more than 800 hosts on it.

Services include remote login (telnet),  file  transfer

(FTP),  mail  (SMTP),  and  numerous  other smaller services

(date,  time,  system  status,  Internet  directory,  etc.).

Reliability is very high.  Speed suffers during peak periods

and telnet can be painful then, but mail always gets through

in a reasonable amount of time, usually minutes.

Every Internet host is supposed to  support  a  command

called  whois that can be used to look up directory informa-

tion.  Failing that, it is possible to telnet to _#s_#r_#i-_#n_#i_#c and

type WHOIS.  Another useful command on _#s_#r_#i-_#n_#i_#c is NIC, which

allows searches of a larger database (which always seems  to

be  out  of  date,  unfortunately).  There is also a printed

directory.

A  great  deal  of  information  can  be  obtained   by

July 6, 1984

- 5 -

anonymous   ftp   (login  anonymous,  password  guest)  from

_#s_#r_#i-_#n_#i_#c.  The specifications for most Internet protocols are

on-line  in the <RFC> directory, and the current host tables

are in the <NETINFO> directory.  RFC is  short  for  Request

for  Comments and is used for historical reasons.  Many RFCs

related to  mail  and  domains  have  also  been  posted  to

net.sources on USENET.

ARPANET and MILNET hosts are all connected to a  subnet

of  IMPs (Interface Message Processors), which are then con-

nected to each other over 56Kbaud dedicated lines plus a few

satellite links (e.g. to Hawaii and Norway).

ARPANET

The original long-haul  packet-switched  computer  net-

work,  funded  by the Advanced Research Projects Agency

(ARPA), which has since changed  its  name  to  Defense

Advanced  Research Projects Agency (DARPA).  It started

as an experiment in 1974 and  worked  so  well  it  had

developed  into  a research utility (run by the Defense

Communications Agency, or DCA) by the end of 1983, when

it  was  split  into MILNET, a production military net-

work, and ARPANET, which reverted to research.   Access

is  limited  officially to organizations doing research

funded by federal money.  Note the name of the  network

is ARPANET; the name of its domain is ARPA.

MILNET

The Defense Data Network the development of  which  was

the  direct goal of the ARPANET research; it split from

the ARPANET in October 1983.  It is  connected  to  the

ARPANET by mail bridges, i.e., gateways that ordinarily

pass only mail and not other traffic  (such  as  remote

login  or  file  transfer).   Non-mail access is mostly

limited to the military and defense contractors.

_#X_#E_#R_#O_#X _#i_#n_#t_#e_#r_#n_#e_#t

The name of the gateway from the ARPANET to  the  Xerox

Grapevine has changed from PARC-MAXC to XEROX since the fol-

lowing mail message was sent.

Date: 6 Jan 84 17:53:05 PST (Friday)

From: Jef Poskanzer <Poskanzer.PA@PARC-MAXC.ARPA>

Subject: Re: The Plethora of Networks

Here's a network you  left  out:  the  XEROX  Internet.

Most  outsiders  tend  to  overlook  the XEROX Internet, for

various reasons:

o   only  a  small  proportion  of  the  traffic   is

gatewayed to or from other networks;

o  what little gatewaying there is gets  done  almost

July 6, 1984

- 6 -

invisibly;

o  the name difficulty.  (I'm told  that  XEROX  used

``Internet'' first, but that doesn't matter much now.)

...

The mail transport mechanism within the XEROX  Internet

is   called   Grapevine.    Grapevine  addresses  look  like

"<user>.<registry>".  If the registry you're sending  to  is

the  one  you  are in, you can leave it off, and the address

becomes merely "<user>".  Registries are  geographic  -  the

two  largest  are "PA" (Palo Alto), for Northern California,

and "ES" (El Segundo), for Southern California.

To send mail in from the  ARPAnet,  the  address  looks

like  "<user>.<registry>@PARC-MAXC".  If the registry is PA,

you can leave it off, giving  "<user>@PARC-MAXC".   This  is

what I mean by invisible gatewaying - to outsiders, it looks

like all 2000 of us Xeroids receive our mail on poor  little

PARC-MAXC.   Not  so  -  it's  just  a gateway.  I think the

source of the confusion is that people are  used  to  expli-

citly  specifying a host for the mail to be delivered to, as

well as a user on that host.  Grapevine's mail  servers  are

politely invisible.

Sending mail out to the  ARPAnet  is  as  easy  as  pi.

"ARPA"   is   just   another   registry,   so   I  just  say

"<user>@<host>.ARPA".  Or if I'm really lazy, I can just say

"<user>@<host>", since anything with at atsign automatically

goes to the ARPAnet.

Date: 19 Mar 84 19:30:09 PST (Monday)

Subject: Re: networks summary

To: jsq@ut-sally.ARPA (John Quarterman)

From: Jef Poskanzer <Poskanzer.PA@PARC-MAXC.ARPA>

That sounds fine to me.  Two quibbles: (1)  The  actual

number  of  Xerox  Internet  Grapevine  mailing addresses is

closer to 5000 than 2000 -  my  counting  method  was  bogus

somehow, and one of the Grapevine implementors corrected me.

(2) There are actually far more devices on the internet than

there  are  Grapevine  addresses,  probably more like 10000;

however,   neither   "ethernet   devices"   nor   "Grapevine

addresses"  corresponds very closely to the usual meaning of

the word "hosts".

_#P_#u_#b_#l_#i_#c _#D_#a_#t_#a _#N_#e_#t_#w_#o_#r_#k_#s

In addition to the summary of five  prominent  networks

below,  Walt  Haas  keeps  a list of contact information for

public data networks worldwide.

July 6, 1984

- 7 -

Date: Mon 9 Jan 84 15:52:34-MST

From: Walt <Haas@UTAH-20.ARPA>

Subject: Yet More Networks

There are five public data  networks  actively  serving

the  US  and  more in the works.  The five national PDNs are

all common carriers, like Greyhound - that is,  anybody  who

pays the fare can use them.  They all provide an X.25 inter-

face, which gives a virtual circuit service -  there  is  as

yet  no international standard for mail or FTP.  All provide

a virtual terminal capability via  the  X.3/X.29  PAD  stan-

dards.   They  all  compete vigorously for business, and I'm

sure I'll hear about it  immediately  if  I  have  left  out

anybody's  capability.  Here (in alphabetical order) are the

five established PDNs:

ADP Autonet

175 Jackson Plaza

Ann Arbor, MI 48106

(313) 769-6800

Besides the US, has satellite links to London,  England

and  Delft, The Netherlands.  Maximum internal speed is 9600

bps.  Nodes are PDP-11s with KMC-11 front  end  microproces-

sors.   Internal  protocol was described to me as derivative

of the old ARPAnet protocols.

CompuServe Incorporated

Network Services Division

5000 Arlington Centre Blvd.

P.O. Box 20212

Columbus, OH 43220

(614) 457-8600

Internal speeds to 56k bps.   Nodes  are  PDP-11s  with

6809 microprocessor front ends.  Internal protocol is DDCMP.

GTE Telenet Communications Corp.

8229 Boone Boulevard

Vienna, VA 22180

(703) 442-1000

Internal speeds to 56k bps.  Nodes are arrays of  6502s

in a redundant, load sharing configuration.  Internal proto-

col  conforms  to  CCITT  Recommendation   X.75.    Supports

automatic recovery of virtual circuit when a node fails dur-

ing a call.  Built by some of the folks from BBN  who  built

the  ARPAnet  originally.   Provides  a  mail service called

Telemail.

July 6, 1984

- 8 -

Tymnet, Inc.

2710 Orchard Parkway

San Jose, CA 95134

(408) 946-4900

Internal speeds to 56k bps.  Nodes are arrays of  "Tym-

net  Engines"  in  a  redundant, load sharing configuration.

The Tymnet Engine is a Tymnet-built 32-bit processor derived

from  the  Interdata  732,  re-engineered for extremely high

MTBF.  Internal protocol is a  unique  Tymnet  design  which

repacketizes inside the network and does flow control at the

byte level, like TCP.  Supports automatic recovery  of  vir-

tual  circuit  when  a node fails during a call.  Provides a

mail service called OnTyme.

Uninet

United Telecom Communications, Inc.

2525 Washington

Kansas City, MO 64108

(816) 221-2444

Internal speeds to 56k bps.  Nodes are  Modcomp  7830s.

Internal  protocol is a Uninet-designed virtual circuit pro-

tocol, on top of HDLC.

_#C_#S_#N_#E_#T

This network consists of three parts logically  treated

as one network: Phonenet, Telenet, and ARPANET.  Phonenet is

constructed of two relay hosts  (currently  csnet-relay  and

rand-relay)  which  poll various other CSNET hosts for mail.

These relays are also ARPANET hosts, and provide mail  gate-

waying  to  the ARPANET.  For those who want better (faster)

service than Phonenet can provide, TELENET  connections  are

available.  This gets a leased line and direct access to the

ARPANET via the Internet.

The address for information about CSNET is

cic@csnet-cic.CSNET

or

cic%csnet-cic.CSNET@csnet-relay.ARPA

so direct questions there rather than  to  Michael  O'Brien,

even  though  he  wrote  the  following useful summary.  The

CSNET directory, containing  much  information  about  CSNET

hosts  and  people  with mailboxes on them, is accessible by

telnet for those with Internet access:  telnet  to  csnet-sh

and log in as user ns with no password, then type help.

July 6, 1984

- 9 -

A good article  on  CSNET  is  ``The  Computer  Science

Research  Network  CSNET:  A  History And Status Report'' by

Douglas Comer, page 747, Communications of the  ACM,  Volume

26, Number 10, October 1983.

Date: Friday,  6 Jan 1984 10:34-PST

Subject: More on CSNET

From: obrien@rand-unix

To expand on CSNET: It is currently funded by the  NSF,

and  expects  to  become self-supporting during the next few

years, based on member fees.  These fees are:

$30,000   commercial sites

10,000   government and not-for-profit

5,000   educational

These fees may be reduced by petitioning for  a  reduc-

tion  in the case of small outfits, and are lower for people

who already have a net connection via Arpanet.

The CSNET membership list as of Dec. 1 shows:

85   Phonenet sites

6   Telenet sites

18   Arpanet sites

4   CSNET-owned hosts

Not all of these sites are operational yet, though most

are.  Phonenet sites are served by two Relay machines, which

call them up nightly to exchange mail.  Text  files  may  be

automatically transferred using MMDF-based mail-receipt pro-

grams, though this is obviously not the best way to do busi-

ness.   Bandwidth  here  is  limited  by the 1200-baud phone

lines as well as by the capacities of the Relays.   Mailing-

list  stuff can be handled OK, but Usenet traffic breaks the

Relays by sheer load.

Telenet sites run TCP/IP on top of  X.25  virtual  cir-

cuits,  using  software developed for CSNET at Purdue.  Per-

sonally I think this is hot stuff.  If your phone bills  are

$1500/month, you can run equivalent traffic over Telenet for

about $1200/month, last time we figured it  out.   And,  you

get  full  Internet  connectivity and services into the bar-

gain.  Because the drop lines from Telenet to the  host  are

really  only 9600, 4800, or 1200 baud dedicated phone lines,

instantaneous bandwidth is not as good as Arpanet, but  it's

July 6, 1984

- 10 -

not  bad.   And, you and the rest of the world will be hard-

put to tell that you're not on Arpanet directly, except  you

don't  have  to  deal  with  the  DoD.  This software really

works, and works well.

Arpanet  sites  run  standard  Arpanet  software  -  no

change.

In addition to simple net  connectivity,  CSNET  brings

the  benefits  of centralized network management.  Basically

this means that if your mail isn't moving, you have  experts

to  scream  to,  and  they  really will work hard to fix the

problem.  There are other benefits such as ongoing mail sys-

tem development, an automatic nameserver, and so forth.

Management of CSNET has recently been transferred  away

from  the  contractor  committees  which  built the net to a

newly-formed Executive Committee, which  is  overseeing  the

move  from  a  research  to a service organization.  The two

relay machines are moving to BBN - it's cheaper  and  easier

to  run  a  single  computer center and communicate via WATS

lines than to spread out the Relay operations.

_#M_#A_#I_#L_#N_#E_#T

This is  evidently  a  star-shaped  phonenet  which  is

reputed  to  use  MMDF  (the  CSNET phonenet software).  For

information   on   a    given    host,    send    mail    to

POSTMASTER%host.MAILNET@MIT-MULTICS.ARPA.

Carnegie-Mellon University                 CARNEGIE.MAILNET

University of Chicago                      UCHICAGO.MAILNET

Dickinson College                          DICKINSON.MAILNET

University of Durham (U.K.)                DURHAM.MAILNET

EDUCOM                                     EDUCOM.MAILNET

Grinnell College                           GRINNELL.MAILNET

Iowa State University                      IOWA-STATE.MAILNET

Massachusetts Institute of Technology      MIT-MULTICS.ARPA

University of Michigan                     UMICH-MTS.MAILNET

New Jersey Institute of Technology         NJIT-EIES.MAILNET

University of Newcastle-upon-Tyne (U.K.)   NEWCASTLE.MAILNET

Northwestern University                    NORTHWESTERN.MAILNET

Stanford University                        STANFORD.MAILNET

Stockholm University QZ Computing Center   QZCOM.MAILNET

Rensselaer Polytechnic Institute           RPI-MTS.MAILNET

_#B_#I_#T_#N_#E_#T

A collection of information on BITNET is maintained  as

July 6, 1984

- 11 -

"dsk:common;bitnet info",  "dsk:common;bitnet arc",  on mit-

mc.ARPA  and  mit-oz  by  Robert  L.  Krawitz,  ZZZ.RLK%MIT-

OZ@MIT-MC.ARPA,  who  also  moderates  the INFO-NETS mailing

list.    The   following    paragraph    is    taken    from

"dsk:common;bitnet info":

Bitnet is a network of computers, mostly  IBM  machines

running  VM/SP which seems to be IBM's main OS for their big

mainframes.  However, there are plenty of VAXen running Unix

or  VMS, as well as a few other machines running random sys-

tems.  There are, as of January 1984, about 150 machines  on

the net, representing about 40 or so (a rough approximation)

different sites, mainly universities.  The central host,  to

whatever degree that is important, is CUNYVM, located at the

City University of New York.  Unfortunately,  I  don't  have

any  contact  names,  so  if you want to find out hard info,

you'll have to look around.

Date: Thu, 5 Jan 84 01:34:37 pst

From: fair%ucbarpa@Berkeley (Erik E. Fair)

Subject: Re:  HUMAN-NETS Digest   V7 #1

This is a network of IBM hosts, and seems to  be  built

along  the  same  lines as the ARPANET (implicit addressing,

dedicated lines, central control) but not all the sites have

the  same  capabilites.   Services  supported: MAIL, and FTP

(for those sites that have RSCS).  Presently is about  50-60

sites.   Founded by CUNY, after they got IBM to cough up the

software that is used in the IBM internal VNET.  I  have  no

idea how fast it goes.  Scope: national.

To address someone on the BITNET from the ARPANET:

mail person%site.BITNET@BERKELEY

BERKELEY's mailer converts this to

G:SITE=PERSON

and it gets sent to UNIX G (in  the  UCB  Computer  Center),

which  in  turn sends it to the IBM 4341 (UCBVMA on the BIT-

NET), and from there it goes where it's supposed to...

_#D_#E_#C _#E_#n_#g_#i_#n_#e_#e_#r_#i_#n_#g _#N_#e_#t_#w_#o_#r_#k

Digital Equipment Corporation (DEC*)  has  an  internal

engineering  network  of more than 2000 hosts, international

scope, and with gateways to several  other  networks.   They

appear  to  have  settled  on the domain name DEC, and there

_________________________

* DEC and DECNET are trademarks of Digital  Equipment

Corporation.

July 6, 1984

- 12 -

will be subdomains.

There is a gateway between the UUCP  mail  network  and

DEC's  network  at decwrl.UUCP and rhea.DEC which has, until

recently, given mail passing  into  the  UUCP  mail  network

return addresses that pretended everything in the DEC domain

was actually in the UUCP domain.   Since  there  are  a  few

dozen site names intersections in the two domains, that pro-

duced a fairly serious routing  problem.   Fortunately,  the

DEC  domain  seems  to  be  nearing  complete implementation

within DEC, and the gateway now produces and accepts reason-

able addresses.  Thus addresses of the form

...!...!decwrl!user%node.DEC

can be used from the UUCP  network  without  worrying  about

mail  getting  misdirected  by those UUCP sites that rewrite

UUCP addresses.

Decwrl is also connected to the TELENET part  of  CSNET

and  thus  serves  as  a  gateway  between the DEC and CSNET

domains and thereby to the rest of the world.

Date: 06-Jan-1984 1256

From: covert@castor.DEC (John Covert)

Subject: A bit more info on Digital's ENET

And second, I'd like to give a little more  information

on  the Digital ENET.  It is composed of systems running our

DECNET software products, first introduced about nine  years

ago.

DECNET is much more than a mail network.  It is a  pro-

duct  built  on  a  layered  network architecture (DNA) with

lower,  non-programmer  accessible  data-link  and   routing

layers, and higher, programmer accessible, session layers.

It is similar to the ISO model on open  systems  inter-

connect.   Since  it  is  older than that model, it does not

correspond exactly, but will, more and more, as time goes by

and as the worldwide networks develop.

At the data-link level it can use synchronous or  asyn-

chronous  lines  of  any speed running DDCMP, public network

lines  running  X.25,  parallel  links   running   protocols

specific to those devices, and Ethernet.  Using gateway pro-

ducts it can create gateway links into an IBM SNA network.

At the user accessible layer, it is  possible  for  any

program  to  open a transparent, full-duplex, channel to any

other program on the same or any other node in the  network.

Programmers  can  take  advantage  of this ``network logical

link'' to build any application they wish.

July 6, 1984

- 13 -

Various Digital supported protocols running on  logical

links are host-to-host terminal connections, allowing a user

at any node to act as an interactive terminal on  any  other

node,  Mail,  the Data Access Protocol, (see next paragraph)

and several others.

The DAP protocol is used to copy files, but it is  much

more than a file copy protocol.  It permits a program on any

system to access a file on any other system as  though  that

file  were a local file.  In fact, VMS and RSX using the DAP

routines buried in RMS permit a nodename to be simply a part

of a file spec used by any program.

DECNET does a bit more than implicit routing;  it  does

dynamic  path routing.  As a result, given sufficient alter-

nate paths, the loss of an intermediate node does not affect

the  operation  of  traffic  currently  routing through that

node.  Dynamic path routing was first made available in DEC-

NET Phase III, offered for sale almost five years ago.

...

Reassigning node  numbers  will  not  be  complete  for

several  months,  and not all systems will upgrade, so there

may be a few systems which require one intermediate hop from

RHEA.   Many  of  these will have definitions on RHEA making

that transparent to the sender (though a recipient would see

the  hop).   The  rest  should  be directly addressable from

RHEA, whether located in the U.S.,  Canada,  the  Caribbean,

Europe (13 countries now), the Middle East, the Far East, or

Australia.  (Remember, IBM is the only computer manufacturer

larger than Digital.)

_#U_#U_#C_#P

The name UUCP is used for two different  protocol  lev-

els,  contributing  to  the  confusion produced by the close

association of UUCP with USENET.

UUCP transport

The name UUCP ( UNIX|#- to Unix CoPy) originally  applied

to  a  transport service used over dialups (1200 or 300

baud) or hardwired lines (usually not higher than  9600

baud)  between  adjacent systems.  (The protocol is not

limited to those speeds and can easily  run  over  much

higher-speed  links,  but only low speed hardware links

were generally available.) While file transfer  appears

to  have been the original intent and main use of UUCP,

there was (and is) no facility for routing beyond adja-

cent  systems  at the transport level.  There has never

been any remote login facility  associated  with  UUCP,

_________________________

|#- UNIX is a trademark of Bell Laboratories.

July 6, 1984

- 14 -

though the cu and tip programs are sometimes used  over

the same phone links.

UUCP mail

This network provides, without a doubt, the worst  mail

service  of  any named: when mail was grafted on top of

UUCP to form a mail network no routing service  and  no

method  of error acknowledgement was provided; thus you

had to guess where to send your mail and  hope  it  got

there.  It is, nonetheless, extremely popular and grow-

ing  rapidly.   This  is  attributable  to   two   cir-

cumstances:  it  is extremely easy to connect a machine

to the UUCP mail network (only one  agreeable  neighbor

is  needed);  and  UUCP  mail  is  very closely tied to

USENET news, which  provides  a  unique  service.   The

development of mail routers is helping, as well.

There is a movement afoot to make the UUCP mail network

into  a  domain  acceptable  for registration with the DARPA

Internet.  This involves (among other things) development of

name  server software, including software to create, distri-

bute, and access routing databases for the UUCP domain.   It

will  also  be  necessary  to subdivide the UUCP domain into

several subdomains.  The coordinator of the effort  is  Mark

Horton, ihnp4!cbosgd!mark, 614-860-4276.

Rick Kiessig, ihnp4!sun!idi!kiessig, plans to publish a

UUCP  mail  directory in May 1984 and is soliciting informa-

tion, including your U.S. Mail address.

Mailing lists similar to  those  long  current  on  the

ARPANET  have  recently seen an upsurge of popularity on the

UUCP mail network  because  they  permit  one  feature  that

USENET newsgroups cannot readily supply: limited access.

_#U_#S_#E_#N_#E_#T

Note that USENET is _#q_#u_#a_#l_#i_#t_#a_#t_#i_#v_#e_#l_#y  different  from  all

the  others:  it  is a news network, with articles sent by a

flooding routing algorithm to all nodes.   While  the  tran-

sport  layer  is  UUCP  for  most links, numerous others are

used, including ethernets, berknets, and the DARPA Internet.

Mail  addresses  have  no meaning on USENET: mail related to

USENET articles is usually sent via UUCP mail.

USENET combines the idea of mailing lists long used  on

the  ARPANET  with bulletin board service as has existed for

many years on TOPS-20 and other systems, adds a  freedom  of

subject  matter  that  could never exist on the ARPANET, and

reaches a more varied constituency.   While  the  result  is

often chaotic and inane ramblings it is quite popular.

There are somewhere around 1000  USENET  hosts  in  the

USA,  Canada,  Australia, Korea, the Netherlands, the United

July 6, 1984

- 15 -

Kingdom, France,  West  Germany,  Denmark,  Sweden,  Norway,

Greece,  Korea,  and  probably  other countries.  There is a

directory and map of USENET maintained by a volunteer (Karen

Summers-Horton, map@cbosgd.UUCP) and posted to the newsgroup

net.news.map monthly.  A list of legitimate  net-wide  news-

groups  is posted to net.news.map monthly.  Other volunteers

keep statistics on the use of the  various  newsgroups  (all

200  or  so of them) and frequency of posting by persons and

sites and  post  them  to  net.news.group.   There  are  two

moderated newsgroups, net.announce and net.announce.newusers

(the current moderator is  Mark  Horton,  mark@cbosgd.UUCP),

which  attempt  to  get important announcements to everyone,

but there is no general central authority for USENET.

The name  USENET  is  derived  from  USENIX,  which  is

derived  from  Unix  User's  Group, as USENIX was once known

before trademark problems caused a name change.  Unix is, of

course, a pun on Multics.

_#O_#t_#h_#e_#r_#s

There are other widespread networks that are  not  pub-

licly  accessible.   IBM operates a very widespread and com-

pletely closed private network (VNET), as doubtless do other

private corporations.  Certain government agencies have net-

works for various purposes.  The government of almost  every

nation  outside  North America controls practically all com-

munications media directly, including  electronic  mail  and

the like.

MCI mail is in such a state of flux at the moment  that

I have not attempted to describe it.

The old TWX and TELEX networks still exist, though they

appear  to  be  being  superseded  by TELETEX (especially in

France).  The most widely distributed electronic  communica-

tions  network  in the world is still the telephone network,

with hundreds of millions of nodes.  The most widely distri-

buted  communications network on this planet is the physical

mail network with probably more than a billion nodes.

July 6, 1984